IONITY GLOBAL

(Pty) Ltd

ESP32-MCP Fleet Platform

Ionity System EDGE Computing

Revamp Phase 1 – Architecture & Interfaces

Monorepo · run_inference sequence · EWP/1 wire contract · Datadog mapping

DOC-2026-10-ESP32MCP-ARCH-P1  |  Version 1.1  |  11 October 2026

PUBLIC

www.ionity.today | ai@ionity.today

# Index – Table of Contents

Phase 1 of the ESP32-MCP revamp fixes the shape of the system before any implementation: the monorepo layout, the end-to-end run_inference sequence from LLM to silicon to Datadog, and the binary wire contract (EWP/1) between the MCP gateway and every ESP32 node. Version 1.1 records the decisions taken at PROCEED.

| Section | Topic | Page |
| --- | --- | --- |
| 1 | Executive summary | 3 |
| 2 | Starting point (v2.2.0 audit) | 3 |
| 3 | Architecture decisions | 4 |
| 4 | Monorepo layout | 5 |
| 5 | run_inference end to end | 6 |
| 6 | EWP/1 wire contract | 7 |
| 7 | Firmware contract (memory safety and RTOS) | 10 |
| 8 | Datadog mapping | 11 |
| 9 | Security baseline | 11 |
| 10 | Decisions taken at PROCEED | 11 |
| 11 | Verification log | 12 |
| A | Appendix A – Document control | 13 |

Page numbers are indicative and may shift slightly between Word versions. Use View > Navigation Pane for live navigation. The engineering source of this document is docs/architecture/PHASE-1-ARCHITECTURE.md in the repository.

# 1. Executive summary

The revamp turns the v2.2 proof of concept into a fleet platform: a TypeScript MCP gateway that speaks the 2026-07-28 MCP specification to Claude and Ollama clients, a compact binary protocol to the boards, an ESP-IDF firmware with dual-core task separation, a browser provisioning portal and end-to-end Datadog tracing.

| Outcome | Result |
| --- | --- |
| Wire size, run_inference request | 29 B on the wire vs 342 B MCP JSON-RPC (11.8× smaller) |
| Wire size, run_inference result | 90 B vs 887 B MCP CallToolResult (9.9×) |
| Telemetry sample | 115 B vs 476 B fw 2.x JSON (4.1×) |
| Type safety | Strict TypeScript contract; 11 compile-time negative checks; builds on TypeScript 5.9.3 and 7.0.2 |
| Firmware contract | C/C++ header generated from the TypeScript source; compiles as C11 and C++17 with -Werror -pedantic |
| Interop | Frames encoded from TypeScript decoded independently by Python cbor2 and by the generated C header |

Phase 1 is additive: it adds packages/protocol/ and docs/architecture/ to the repository and moves nothing. The folder restructure is applied only after approval.

# 2. Starting point (v2.2.0 audit)

| Component | Today | Gap against the target |
| --- | --- | --- |
| Firmware 2.1.1 | Arduino core 3.3.11 (pioarduino), ArduinoJson 7, PubSubClient | Board parses full MCP JSON-RPC; tools run synchronously in loop(); MQTT 3.1.1 only; 4 KB JSON buffers |
| Fleet host 2.2.0 | Python FastAPI, MCP JSON-RPC 2025-06-18, 18 fleet tools, SQLite, alerts, LAN DNS | No traces; Datadog via HTTP API only; device bridge forwards JSON-RPC verbatim |
| Flasher 2.0 | React 19, esptool-js 0.7, Web Serial, ionity-prov/1, keep-NVS flashing | No terminal, no device UUID, no EWP awareness |
| Plan v3 | NetObserve tools on the board (DNS, local devices) | Tool ids 0x40–0x5F reserved in EWP/1 |

# 3. Architecture decisions

| # | Decision | Rationale |
| --- | --- | --- |
| D1 | Gateway in Node 24 LTS + TypeScript on MCP SDK v2 (@modelcontextprotocol/server 2.3.x, spec 2026-07-28). The Python host becomes services/fleet-host and is retired feature by feature. | Stateless MCP (no initialize, no session id) lets gateway instances scale out without affinity; one language with the portal; existing tools keep working during migration. |
| D2 | EWP/1: 16-byte little-endian header + one CBOR body with 1-byte integer keys. | Core 0 routes frames without a CBOR parser; 29 B vs 342 B per request. |
| D3 | MQTT 5 primary, WSS secondary, same frames. Clean sessions; message expiry = request deadline. | Broker fan-out, Last Will presence, per-device ACLs; correctness never depends on an MQTT 5 feature. |
| D4 | Firmware: ESP-IDF C++, one tree built by PlatformIO (pioarduino) and idf.py in CI; IDF 5.5 now, 6.x-ready. | Full sdkconfig control; IDF 6.0 removed legacy drivers and moved esp-mqtt and cJSON to the Component Registry. |
| D5 | Identity: hwid esp32-<MAC> on the wire and in ACLs; provisioned UUID (RFC 9562) as logical asset id. | Keeps fw 2.x ids and topics; the UUID survives a board swap. |
| D6 | Fleet-shaped MCP surface: generic tools with a device argument, never one tool set per board. | 100k boards cannot become 100k tool lists; tools/list stays cacheable. |
| D7 | Datadog via the Agent (APM, DogStatsD, Logs) with dd-trace 6.x; device spans rebuilt from device timings. | End-to-end traces without device clocks; bounded metric cardinality. |
| D8 | Fail-safe actuation: MRTR confirmation for risky calls (D10), dead-man leases, PRIORITY FAILSAFE broadcast. | An LLM-driven relay must never stay ON because a session died. |
| D9 | Broker: Mosquitto now, a cluster later. Lab, pilots and first customers on Mosquitto (South Africa or the customer's site). A cluster – EMQX with a commercial licence or a managed service in an approved region – when failover is needed or the fleet passes ~10 000 boards, chosen from measured traffic. | Free for commercial use, already in the repo, covers MQTT 5, LWT, ACLs and TLS. One server without automatic failover. The gateway speaks standard MQTT, so the switch is configuration, not a rewrite. |
| D10 | Confirm every risky command from an LLM/MCP client that changes hardware state or settings (MRTR or an approval link). Exceptions: operator-defined rules are approved once at setup; entering FAILSAFE is immediate, leaving it needs confirmation. | The owner's rule. Stopping things is never delayed; starting things always has a human in the loop. |
| D11 | Restructure after Phase 3. Phases 1–3 only add packages/, apps/portal/ and docs/architecture/. | Nothing in the field or in the lab scripts breaks mid-revamp. |
| D12 | npm workspaces for the TypeScript side; pnpm + turbo reconsidered at the restructure. | No new tool on the lab PCs today; the layout is the same either way. |

# 4. Monorepo layout

Target layout. Arrows (←) show where today’s folders move; entries marked new are created in their phase.

Esp32-MCP/                      github.com/dennisGIonity/Esp32-MCP (copy → Ionity-Global-Pty-Ltd)

├─ apps/

│  ├─ portal/                   React 19 · Vite 8 · Tailwind 4 · esptool-js · xterm.js  ← flasher/  (Phase 2)

│  │  ├─ src/features/device/      DeviceConsole (core component), connection, board card

│  │  ├─ src/features/provision/   ionity-prov/2 form (prov/1-compatible), pairing checklist

│  │  ├─ src/features/console/     xterm.js console, secret scrubbing, Save log

│  │  ├─ src/features/flash/       esptool-js through a port lease, keep-NVS, SHA-256 checked

│  │  ├─ src/lib/serial/           single-owner Web Serial session (DTR/RTS-safe, re-enumeration)

│  │  └─ src/sim/                  ?simulate=v1|v2|v2-usb boards for demos and end-to-end tests

│  └─ gateway/                  Node 24 LTS · TypeScript · MCP SDK v2                   (new)

│     ├─ src/mcp/                  Streamable HTTP + stdio, tool registry, MRTR confirms

│     ├─ src/tools/                run_inference · set_actuator · read_telemetry · …

│     ├─ src/link/                 mqtt.ts (MQTT 5) · ws.ts (WSS) · rpc.ts (rid, deadline)

│     ├─ src/registry/             HELLO cache, presence (STATUS/LWT), hwid ↔ uuid ↔ label

│     ├─ src/devops/               OTA campaigns, build triggers, ELF archive

│     ├─ src/observability/        dd-trace, span rebuild, DogStatsD, crash symboliser

│     └─ src/api/                  REST for the portal: defaults, enrolment, manifest

├─ firmware/

│  ├─ esp32-node/               ESP-IDF C++ · PlatformIO (espidf) + idf.py              (new)

│  │  ├─ platformio.ini · CMakeLists.txt · idf_component.yml · partitions.csv

│  │  ├─ sdkconfig.defaults · sdkconfig.defaults.esp32s3 · sdkconfig.defaults.esp32c6

│  │  ├─ main/main.cpp             boot, NVS, task topology (Core 0 net / Core 1 tools)

│  │  ├─ components/ewp/           frame codec (tinycbor) + ewp_protocol.h (generated)

│  │  ├─ components/mcp_server/    MCPServer.h: registry, job pool, worker tasks

│  │  ├─ components/net/           wifi_mgr · mqtt_link (MQTT 5) · ws_link · sntp

│  │  ├─ components/tools/         the four edge tools + identify · get_device_info

│  │  ├─ components/edge_ai/       ESP-DL / TFLM runners, PSRAM arenas, model manifest

│  │  ├─ components/actuators/     LEDC / SDM / GPIO channels, leases, FAILSAFE latch

│  │  ├─ components/provisioning/  ionity-prov/2 over UART/USB-CDC, NVS v2 + v1 migration

│  │  ├─ components/crash/         core-dump summary → CRASH frame on next boot

│  │  ├─ components/http_mcp/      optional LAN JSON-RPC MCP (espressif/mcp-c-sdk)

│  │  └─ test/                     Unity tests + golden vectors from packages/protocol

│  ├─ legacy-arduino/           fw 2.x sketches, frozen (bug fixes only)  ← firmware-arduino/

│  └─ tools/                    build.py → images + manifest + ELF archive ← firmware/build.py

├─ packages/

│  ├─ protocol/                 @ionity/ewp — EWP/1 wire contract                 (Phase 1)

│  ├─ provisioning/             @ionity/prov — NVS record v2 schema               (Phase 2)

│  └─ tsconfig/                 strict presets (the protocol tsconfig, promoted)

├─ services/

│  ├─ fleet-host/               Python FastAPI 2.2, retired step by step ← server/ + dashboard/

│  └─ network-sentinel/                                      ← modules/network-sentinel/

├─ devices/pi-agent/            unchanged

├─ infra/

│  ├─ mosquitto/                lab broker, ACL generated from CHANNEL_POLICY ← infra/mosquitto/

│  ├─ broker-cluster/           later (D9): EMQX (licensed) or managed MQTT, chosen from real traffic

│  ├─ datadog/                  agent (APM + DogStatsD + logs), dashboards, monitors

│  ├─ pki/                      fleet CA + device certificates (no keys in git)

│  └─ compose/                  lab stack: broker + gateway + fleet-host + agent ← docker-compose.yml

├─ tools/

│  ├─ ewp-emulator/             N virtual EWP/1 boards for load tests and CI

│  ├─ scripts/                  lab start/stop/status, provision.py, net_watch.py ← scripts/

│  └─ packaging/                testers + POC kits                               ← packaging/

├─ docs/                        + architecture/ · adr/

├─ .github/workflows/           npm workspaces · pytest · idf.py matrix (S3, C6) · vectors · header drift

└─ package.json (npm workspaces, D12) · tsconfig.base.json · .nvmrc (24)

Migration rules. fw 2.x JSON topics (ionity/<site>/<id>/…) and EWP topics (ionity/ewp1/…) run side by side on one broker, so mixed fleets keep working. The Python host keeps its MCP endpoint until the gateway reaches tool parity.

# 5. run_inference end to end

An LLM asks a board to classify a vibration window. The gateway translates the MCP call into a 29-byte frame; Core 0 routes it without parsing CBOR; Core 1 runs the model; the 90-byte result returns to the LLM while traces, metrics and logs flow to Datadog.

Source: docs/architecture/run_inference_sequence.mmd (renders with Mermaid 11+).

## 5.1 Device spans without a device clock

The board has no RTC, so its timestamps are never used. The gateway records publish time t0 and result arrival t3; the device reports queueUs and execUs from esp_timer_get_time(). One-way delay d = max(0, (t3 − t0 − queueUs − execUs) / 2); the esp32.queue span starts at t0 + d and esp32.tool.exec follows it, created with tracer.startSpan(name, { childOf, startTime }) and closed with span.finish(endTime).

## 5.2 Trace continuity and unhappy paths

- The 2026-07-28 spec carries W3C traceparent in _meta. dd-trace 6.20 auto-instruments only the v1 SDK package (via _meta._dd_trace_context), so the gateway extracts both keys itself.

- BUSY (all 4 job slots in use) is answered immediately with retryAfterMs.

- No result by deadline + 1.5 s: the gateway publishes CANCEL and returns isError: true, a tool error the model can reason about.

- A job still queued when its budget runs out is answered DEADLINE_EXCEEDED and never executed.

# 6. EWP/1 wire contract

Source of truth: packages/protocol (TypeScript, package @ionity/ewp). generated/ewp_protocol.h is produced from it; no number is ever hand-edited on either side.

## 6.1 Header – 16 bytes, little-endian, naturally aligned

| Offset | Size | Field | Notes |
| --- | --- | --- | --- |
| 0 | 1 | magic | 0xE5 – rejects legacy JSON or a bare CBOR map at byte 0 |
| 1 | 1 | version | 1 |
| 2 | 1 | type | REQ, ACK, RES, ERR, CANCEL, TELEMETRY, EVENT, LOG, CRASH, HELLO, STATUS |
| 3 | 1 | flags | ACK_REQUESTED, BROADCAST, PRIORITY, BUFFERED, TRUNCATED – unknown bits rejected |
| 4 | 4 | rid | (gateway instance << 24) ¦ counter; per-boot sequence on device frames |
| 8 | 1 | tool | tool id, 0 for non-tool frames |
| 9 | 1 | status | 0 OK, 1 ACCEPTED, 0x10+ errors |
| 10 | 2 | deadlineMs | REQ budget relative to receipt – no clock sync needed |
| 12 | 2 | bodyLen | 16 + bodyLen must equal the payload length |
| 14 | 2 | reserved | must be 0 |

## 6.2 Topics – ionity/ewp1/<site>/<hwid>/<channel>

| Channel | Direction | QoS | Retained | Frames |
| --- | --- | --- | --- | --- |
| dn | gateway → device | 1 | – | REQ, CANCEL (message expiry = deadline) |
| rs | device → gateway | 1 | – | ACK, RES, ERR |
| tm | device → gateway | 0 | – | TELEMETRY |
| ev | device → gateway | 1 | – | EVENT, LOG, CRASH |
| hello | device → gateway | 1 | yes | HELLO (capabilities) |
| st | device → gateway | 1 | yes | STATUS – also the Last Will (OFFLINE) |
| _all/dn | gateway → all | 1 | – | BROADCAST requests only; devices jitter replies 0–2 s |

## 6.3 Tool registry

| Id | MCP name | Wire args → result | Flags |
| --- | --- | --- | --- |
| 0x01 | get_device_info | {} → HELLO body | read-only |
| 0x02 | read_telemetry | {sections?} → TELEMETRY body | read-only |
| 0x03 | run_inference | {model, input: LIVE ¦ INLINE ¦ BLOB, topK?, threshold?} → {classes [[cls, p]], score?, positive, stages, arenaPeakBytes} | read-only, long-running |
| 0x04 | set_actuator | {channel, value 0..1, rampMs?, leaseMs?} → {applied, previous, leaseRemainingMs?} | destructive |
| 0x05 | set_state_mode | one variant per mode → {previous, current, effectiveInMs, latched} | destructive |
| 0x06 | identify | {durationMs?} → {} | – |
| 0x40–0x5F | NetObserve (plan v3) | reserved | – |

## 6.4 Strict TypeScript interface (condensed)

// packages/protocol (@ionity/ewp) - condensed view; full source: 9 files in packages/protocol/src

// ---- wire scalars: range-checked brands, a bare `number` can never reach the MCU ----------------

type Brand<T, B extends string> = T & { readonly [brand]: B };

export type U8 = Brand<number, 'u8'>;  export type U16 = Brand<number, 'u16'>;

export type U32 = Brand<number, 'u32'>; export type I32 = Brand<number, 'i32'>;

export type F32 = Brand<number, 'f32'>;                 // finite, Math.fround()-ed

export type Unit = Brand<number, 'unit'>;               // finite, 0..1 (duty, probability)

export type Utf8<N extends number> = Brand<string, `utf8<=${N}`>;           // UTF-8 BYTES, no NUL

export type Bytes<N extends number> = Brand<Uint8Array, `bytes<=${N}`>;

export type FixedBytes<N extends number> = Brand<Uint8Array, `bytes==${N}`>;

// ---- identifiers: `as const` (erasable, generated 1:1 into ewp_protocol.h) ----------------------

export const MsgType = { REQ: 0x01, ACK: 0x02, RES: 0x03, ERR: 0x04, CANCEL: 0x05,

TELEMETRY: 0x10, EVENT: 0x11, LOG: 0x12, CRASH: 0x13, HELLO: 0x20, STATUS: 0x21 } as const;

export const ToolId = { GET_DEVICE_INFO: 0x01, READ_TELEMETRY: 0x02, RUN_INFERENCE: 0x03,

SET_ACTUATOR: 0x04, SET_STATE_MODE: 0x05, IDENTIFY: 0x06 } as const;   // 0x40-0x5F: plan v3

export const StateMode = { ACTIVE: 0, STANDBY: 1, INFERENCE_ACTIVE: 2,

LOW_POWER_SLEEP: 3, FAILSAFE: 4 } as const;                          // = fw 2.x NVS values

export const InputSource = { LIVE: 0, INLINE: 1, BLOB: 2 } as const;

export type MsgType = (typeof MsgType)[keyof typeof MsgType];         // same for every enum

// ---- 16-byte little-endian header: memcpy() into ewp_header_wire_t on the ESP32 ----------------

export interface FrameHeader {

readonly magic: 0xe5;            readonly version: 1;

readonly type: MsgType;          readonly flags: U8;        // unknown bits => reject

readonly rid: U32;                                          // (gateway << 24) | counter

readonly tool: ToolId | 0;       readonly status: Status;

readonly deadlineMs: U16;                                   // relative: no clock sync needed

readonly bodyLen: U16;           readonly reserved: 0;

}

// ---- registry: header.tool and body are correlated by construction -----------------------------

export interface ToolContract {

readonly [ToolId.GET_DEVICE_INFO]: { args: EmptyBody;         result: HelloBody };

readonly [ToolId.READ_TELEMETRY]:  { args: ReadTelemetryArgs; result: TelemetryBody };

readonly [ToolId.RUN_INFERENCE]:   { args: RunInferenceArgs;  result: InferenceResult };

readonly [ToolId.SET_ACTUATOR]:    { args: SetActuatorArgs;   result: SetActuatorResult };

readonly [ToolId.SET_STATE_MODE]:  { args: SetStateModeArgs;  result: SetStateModeResult };

readonly [ToolId.IDENTIFY]:        { args: IdentifyArgs;      result: EmptyBody };

}

export type RequestFrame = { readonly [T in ToolId]: {

readonly header: FrameHeader & { type: typeof MsgType.REQ; tool: T };

readonly body: ToolContract[T]['args'];

} }[ToolId];

export type DownlinkFrame = RequestFrame | CancelFrame;                 // all a gateway may send

export type UplinkFrame = AckFrame | ResultFrame | ErrorFrame | TelemetryFrame

| EventFrame | LogFrame | CrashFrame | HelloFrame | StatusFrame;    // all a device may send

export interface ResultBody<R> {

readonly timing: { queueUs: U32; execUs: U32; heapDelta: I32 };     // -> Datadog spans

readonly result: R;

}

// ---- run_inference: the spec's `input_frame: any` becomes a bounded union ----------------------

export type InferenceInput =

| { readonly source: typeof InputSource.LIVE; readonly windowMs: U16 }

| { readonly source: typeof InputSource.INLINE; readonly dtype: TensorDType;

readonly shape: TensorShape; readonly data: Bytes<2048> }   // prod(shape)*dtype == bytes

| { readonly source: typeof InputSource.BLOB; readonly blobId: U32;

readonly sha256: FixedBytes<32> };

export interface RunInferenceArgs {

readonly model: U8;                   // handle from HELLO; gateway maps "vibration_anomaly"

readonly input: InferenceInput; readonly topK?: U8; readonly threshold?: F32;

}

export interface InferenceResult {

readonly model: U8; readonly modelHash: FixedBytes<8>;              // -> labels + version

readonly classes: readonly (readonly [cls: U8, p: Unit])[];          // top-k, descending

readonly score?: F32; readonly positive: boolean;

readonly stages: readonly [preUs: U32, invokeUs: U32, postUs: U32];

readonly arenaPeakBytes: U32;                                        // PSRAM high-water mark

}

// ---- actuation and modes: unsafe states are unrepresentable ------------------------------------

export interface SetActuatorArgs {

readonly channel: U8; readonly value: Unit; readonly rampMs?: U16;

readonly leaseMs?: U32;                     // dead-man: reverts to the channel's safe value

}

export type SetStateModeArgs =

| { mode: typeof StateMode.ACTIVE } | { mode: typeof StateMode.STANDBY }

| { mode: typeof StateMode.INFERENCE_ACTIVE; model: U8; periodMs?: U16 }

| { mode: typeof StateMode.LOW_POWER_SLEEP; durationS: U32 }       // 5..86400, reply first

| { mode: typeof StateMode.FAILSAFE; reason?: Utf8<48> };           // latched in NVS

// ---- telemetry: one ESP-IDF call per field (+ sensors, inference, tasks in the full file) ------

export interface TelemetryBody {

readonly seq: U32; readonly uptimeS: U32; readonly unixS?: U32; readonly ageMs?: U32;

readonly mode: StateMode;

readonly heap?: { internalFree: U32; internalMinFree: U32; internalLargest: U32;

psramFree?: U32; psramMinFree?: U32; psramLargest?: U32; dmaFree?: U32 };

readonly net?: { rssiDbm?: I8; channel?: U8; txFail: U32; reconnects: U16; uplinkQueued: U8 };

readonly cpu?: readonly [core0: U8] | readonly [core0: U8, core1: U8];   // C6 = 1 core

readonly actuators?: readonly Unit[];

readonly ext?: ReadonlyMap<U8, F32>;       // custom metrics; names sent once in HELLO

}

// ---- CBOR keys: 1 byte each, checked by the compiler ------------------------------------------

export const RunInferenceArgsKey =

defineKeys<RunInferenceArgs>()({ model: 1, input: 2, topK: 3, threshold: 4 });

// defineKeys<T>() fails to compile on a missing/extra field, a duplicate key or a key > 23.

## 6.5 Measured frame sizes

| Frame | EWP/1 | JSON equivalent | Ratio |
| --- | --- | --- | --- |
| run_inference request | 29 B | 342 B (MCP tools/call with _meta) | 11.8× |
| run_inference result | 90 B | 887 B (MCP CallToolResult) | 9.9× |
| run_inference request vs fw 2.x MQTT envelope | 29 B | 184 B | 6.3× |
| Telemetry sample | 115 B | 476 B (fw 2.x JSON) | 4.1× |

Request on the wire: e5 01 01 01 2a 00 00 01 03 00 b8 0b 0d 00 00 00 | a3 01 03 02 a2 01 00 02 19 03 e8 03 03 – header (REQ, ACK_REQUESTED, rid 0x0100002A, tool 3, deadline 3000 ms, body 13 B) followed by {1: 3, 2: {1: 0, 2: 1000}, 3: 3}.

## 6.6 Errors and compatibility

- Every device-side failure becomes a tool result with isError: true and the detail text; JSON-RPC errors are reserved for malformed MCP requests.

- BUSY, DEADLINE_EXCEEDED, NOT_READY and RATE_LIMITED are retryable and carry retryAfterMs.

- Ids, keys and header offsets are append-only; a breaking change bumps version and HELLO advertises [min, max].

- Decoders ignore unknown CBOR keys but reject unknown header flags; float fields may arrive as CBOR integers; non-finite floats are rejected.

# 7. Firmware contract (memory safety and RTOS)

Binding rules for Phase 3. Each one closes a failure mode seen on ESP32 fleets.

- No allocation on the hot path. 4 × 4 KiB job slots are allocated once at boot; net_task copies a payload into a free slot or answers BUSY.

- Core 0 never blocks on Core 1. xQueueSend(job_q, …, 0) – back-pressure is an error frame, not a stalled network task.

- One publisher. Only net_task publishes; esp-mqtt MQTT 5 publish properties are a one-shot setting before each publish.

- Memory placement. Tensor arenas, log rings and the frame pool in PSRAM; DMA buffers in internal RAM; any task that writes NVS or flash keeps its stack in internal RAM.

- Watchdog-safe inference. Long stages yield so IDLE1 runs; the worker subscribes to the task watchdog explicitly.

- Fragmentation is measured. Telemetry reports the largest free block; below one frame the board can no longer receive a request.

- Validation order. Length, magic, version, flags, reserved, 16 + bodyLen, size limit, type allowed on topic, tool registered, rid dedupe, free slot.

- No stale actuation. Clean sessions, message expiry, device-side deadline, rid dedupe window, leases that revert to safe values, FAILSAFE latched in NVS.

- Chip-aware. ESP32-S3 has no DAC (SDM or LEDC + RC instead). ESP32-C6 is single-core and the targeted modules have no PSRAM: tasks run unpinned and HELLO reports psramMb = 0.

# 8. Datadog mapping

| Metric | Type | Source | Tags |
| --- | --- | --- | --- |
| esp32.mcp.tool_execution_ms | distribution | RES execUs | tool, model, fw, chip, site, status |
| esp32.mcp.queue_wait_ms | distribution | RES queueUs | tool, site |
| esp32.mcp.rpc_rtt_ms | distribution | publish → RES | tool, transport, site |
| esp32.mcp.errors | count | ERR frames | tool, status |
| esp32.heap.free / .min_free / .largest_block | distribution | TELEMETRY heap | site, fw, chip |
| esp32.heap.fragmentation | distribution | 1 − largest / free | site, fw |
| esp32.psram.free | distribution | TELEMETRY heap | site, fw |
| esp32.wifi.rssi | distribution | TELEMETRY net | site |
| esp32.cpu.load | distribution | TELEMETRY cpu | site, core |
| esp32.task.stack_free_min | distribution | TELEMETRY tasks | task, fw |
| esp32.inference.arena_peak_bytes | distribution | RES result | model, fw |
| esp32.fleet.online | gauge | registry | site, fw |

Cardinality rule. No device_id tag on metrics. Dropping it from a gauge would make 1 000 boards overwrite one series, so per-device values are sent as distributions (fleet percentiles per firmware and site). A bounded allow-list of up to 50 watched boards may emit per-device gauges. device_id lives on traces and logs.

Logs and crashes. LOG frames become Datadog logs stamped with dd.trace_id and dd.span_id looked up from rid. CRASH frames are symbolised with addr2line against the archived ELF matched by elfSha and sent with error.kind GuruMeditation, so Error Tracking groups crashes by stack.

# 9. Security baseline

- Per-device X.509 (mTLS) to the broker; on ESP32-S3 the private key is protected by the Digital Signature peripheral. ACL: write only below the board’s own hwid.

- Production boards: secure boot v2, flash encryption and HMAC-based NVS encryption. Consequence for Phase 2: provisioning always goes through the firmware (ionity-prov/2 over serial); the portal never writes a raw NVS image.

- Gateway: OAuth per the MCP authorization specification for remote clients, bearer token for LAN stdio, destructive tools gated by policy.

# 10. Decisions taken at PROCEED

| Question in v1.0 | Answer (11 October 2026) | Recorded as |
| --- | --- | --- |
| Gateway runtime: Node/TypeScript or Python FastMCP | Node/TypeScript | D1 |
| Fleet broker: EMQX (proposed) or a managed service | Mosquitto now; a cluster later (licensed EMQX or managed, approved region) | D9 |
| Confirmation policy | Confirm every risky command (operator rules approved at setup; FAILSAFE entry immediate) | D10 |
| Restructure timing | After Phase 3 | D11 |

Correction to v1.0. v1.0 proposed EMQX for the fleet tier without its licence terms. EMQX 5.9 and later are published under the Business Source License 1.1: a single node is free to use, but not when it is offered as a service or embedded in a product that is sold, and clustering always needs a commercial licence. D9 therefore starts on Mosquitto and leaves the cluster choice to the point where failover is needed. AWS IoT Core has no Cape Town (af-south-1) endpoint and HiveMQ Cloud Starter is hosted in the EU/US only – inputs for the approved-region question when the time comes.

# 11. Verification log

| Check | Result |
| --- | --- |
| packages/protocol type-check (strict, exactOptionalPropertyTypes, noUncheckedIndexedAccess, erasableSyntaxOnly) | Pass – TypeScript 5.9.3 and 7.0.2 |
| Compile-time contract tests (11 @ts-expect-error negatives + positive cases) | Pass |
| Runtime guard tests (ranges, finite floats, UTF-8 byte bounds, topic injection, layout) | 9 / 9 pass – Node 22 |
| Generated ewp_protocol.h | Compiles as C11 and C++17 with -Wall -Wextra -Werror -pedantic; static_asserts on size and offsets |
| Cross-implementation decode of REQ / RES / TELEMETRY frames | Python cbor2 and the generated C++ header agree with the TypeScript encoding |
| Sequence diagram | Renders with @mermaid-js/mermaid-cli 12.0.0 |
| v1.1: packages/protocol inside the npm workspace (exports ./src/index.ts for the portal and gateway) | Type-check and 9 / 9 runtime tests pass |

# Appendix A – Document control

| Field | Value |
| --- | --- |
| Document ID | DOC-2026-10-ESP32MCP-ARCH-P1 |
| Title | ESP32-MCP revamp – Phase 1: Architecture & Interfaces |
| Version / date | 1.1 – 11 October 2026 (SAST) |
| Status | Approved at PROCEED (11 October 2026) |
| Classification | PUBLIC |
| Owner | Johan Wilhelm van Antwerp ¦ Ionity (Pty) Ltd ¦ Ionity Global (Pty) Ltd ¦ AEDI |
| Prepared by | Claude (Anthropic), acting as lead architect for Ionity Global |
| Repository | github.com/dennisGIonity/Esp32-MCP – docs/architecture/PHASE-1-ARCHITECTURE.md, packages/protocol/ |
| Governance / licence | Policy 986 AED ¦ License AED 900 ¦ CC BY-NC-SA 4.0 where stated |
| Rights | (c) 2018-2026 Antwerp Designs ¦ Ionity (Pty) Ltd ¦ Ionity Global (Pty) Ltd – All Rights Reserved – TM2 |
| Web | www.ionity.today ¦ www.ionity.world ¦ Ref: www.ionity.co.za |
|  | Building Tomorrow, Today. ¦ Anything is Possible with God. |

## Change log

| Version | Date | Change |
| --- | --- | --- |
| 1.0 | 11 Oct 2026 | First issue for PROCEED |
| 1.1 | 11 Oct 2026 | Decisions D9–D12 and section 10 recorded; EMQX licence correction; monorepo tree updated to the Phase 2 portal layout, infra/broker-cluster and npm workspaces; broker label in the sequence diagram |